Adapt-A-Mast

Installation Instructions

Page 1

The following instructions apply to the "Standard" and "Standard Tilting" Adapt-A-Mast.

The Standard Adapt-A-Mast is supplied with 2 wall brackets, U bolts and a DL600 winch.  The mast is assembled and is ready for installation.

Steps for installation are recommended in the following order:

1)	Decide where the mast has to be placed.

2)	After unpacking the mast and brackets

a)	Lay aside the winch

b)	Loosely fit the mounting brackets to the mast in the approximately correct positions, (normally one between the winch mounting and bottom pulley, and the other a little below the bottom of the two top bolts).  These can be adjusted to suit individual requirements.

3)	Raise the mast into the position required, and finally adjust the brackets if necessary.  (When the final position of both brackets is decided, tighten the "U" bolts securing the brackets to the mast).

4)	Mark and drill the wall using the mast and brackets as a template. (Note! the correct drill relative to the size of the rawl bolts must be used).

5)	Secure the assembly to the wall using 4 rawl bolts or through bolts. (Please note: these bolts are not provided).

6)	Fit the winch to the winch plate with the bolts already fitted. You will require a 1/2" AF or l3mm spanner to tighten these -be careful not to overtighten them.

After fitting the winch to the mast, finally fit the cable to the winch (the instructions for fitting the cable and handle to the winch are enclosed in the winch box).

7)	Once all fixings (wall, mast and winch) are secure, wind the handle and raise the mast to its full height (Note: the bottom bolt of the two bolts at the top of the mast is a stop and the inner mast cannot be raised above it.

Note!  Very little effort is required to raise the mast from low vertical to full height.

Adapt-A-Mast

Page 2

Additional Instructions for Tilting Model

The following items are included in the kit for the tilting Adapt-A-Mast

A)	Winch and wall bracket

B)	Cable and grips

C)	Top pulley

D)	Ring and plate plus U bolt, nuts and washers

E)	2 angles and 1 x M12 bolt 90mm long, with a nut and washer for securing the mast to the ground (normally to a patio or paving slab)

To Fit the Tilting System.

Attend to items 1 to 7 on Page 1, but first of all fit the 2 angles to the bottom of the mast using the 2 centre holes in the brackets, and the bolt provided.

Once the ground tilting brackets have been fitted to the mast then the mast can be raised to its normal position, when the top pulley can be placed and the position marked. (Note!! the position for the pulley should be below the top wall bracket).

The winch should be bolted to the wall bracket, before the bracket is positioned to the wall.

Points to look out for are as follows

a)	The winch and bracket must be able to operate - remember the handle has to turn.

b)	The top pulley may need to be slightly angled to suit the position chosen for the winch.

Note!!   When fitting the tilting brackets and assembly always determine the position of the wall brackets before securing the hinged angles to the slab (it is best to have the angles secured to the mast when mounting the mast).

With sensible use and occasional maintenance this mast will give many years of excellent service.

Please do not hesitate to contact us if you need any further help or guidance.